Periodic Survey Report Template:

Anchoring and winching systems

Excluding winches relating to lifting appliances covered by Rule Parts 40D.67(12)-(13) and Part 49

| Survey for compliance of: | Survey findings | Add photos |
| --- | --- | --- |
| Anchors |  |  |
| Condition (From inspection of anchors for deformation, wear, corrosion, and wear in anchor head holes, pin or shackle connections). |  |  |
| Adequacy of anchors (For anchors 75kg or more, from comparing markings on anchor and accessories with the certificate of test.) |  |  |
| Anchor cables, chains and ropes |  |  |
| Condition of chain (From inspection for wear and corrosion and measurement of the link diameter.) |  |  |
| Adequacy of chain cables (For chain cables 12.5mm or more in diameter, from comparing markings with data on the certificate of test) |  |  |
| Anchor wire rope condition (From inspection for broken wires, wastage, corrosion, deformation, and other indications of weakness) |  |  |
| Fiber Rope condition (From inspection sunlight damage, abrasions, hockles and other indications of weakness). |  |  |
| Fairleads and samson posts |  |  |
| Condition (From inspection for deformation, diminution, corrosion, cracking, and secureness of attachment.) |  |  |
| Windlasses and capstans |  |  |
| Secureness (From inspection of the secureness of the attachment to the deck.) |  |  |
| Adequate size and power (From inspection that the device is suitable for the size of chain and anchor attached.) |  |  |
| Condition (From inspection for damage of any condition of any moving parts.) |  |  |
| Braking and release (From inspection that devices enable the release, braking and securing of cable as required by relevant rules.) |  |  |
| Safe manual operation (For devices that enable hand and power operation, from inspection that manual drive is not operational in power mode.) |  |  |
| Anchoring system operation |  |  |
| Deployment readiness (From checking that primary anchoring system is ready to deploy at all times and, where two anchors are required to be carried, that the secondary system is capable of been readily deployed.) |  |  |
| Effectiveness (From testing of the anchor lowering, brake, cable-stoppers/claws/securing method when the cable is at required length, and from testing of retrieval.) |  |  |
| Winches |  |  |
| Adequacy for the load (From inspection of wires and warps for adequacy for size of anticipated loads, and of the means of preventing overloading and accidental release of load) |  |  |
| Adequacy of guarding (From inspection of adequacy of guarding and fencing of moving parts of winches, of warp and chain leads, and of warps between lead rollers and sheaves and rollers) |  |  |
| Safety of manually operated guiding (Where applicable, from inspection that operating wheels have no dangerous open spokes or protrusions and are capable of being disengaged when the warps are paying out.) |  |  |
| Location of operating controls (From inspection that location of winch controls allows unimpeded operation and an unobstructed view of the winch and working area.) |  |  |
| Dual control interlocks (Where a winch is provided with local and remote controls, from test that simultaneous operation is prevented.) |  |  |
| Stoppering devices (Where applicable, from inspection of suitability of chains and other devices for stoppering off trawl boards.) |  |  |
| Arrest and hold effectiveness (From inspection and test of winches effectiveness at arresting and holding the safe working load.) |  |  |
| Accidental release safety (From testing that winches have an effective means of preventing the accidental unsafe release of a load.) |  |  |
| Brakes (From inspection of winch drum brakes adequate on all required drums and of the accessibility and ease of adjusting the brakes.) |  |  |