Part 199 Survey Report for ships 400 GT or more

| Name of vessel |  |  | MNZ/MSA Number |  |  |
| --- | --- | --- | --- | --- | --- |
| Name of operation |  |  |  |  |  |
| Place of survey |  |  |  |  |  |
| Date of survey |  | Surveyor Name |  |  |  |
| Stamp |  | Signature |  |  |  |
| Report date |  | Report number |  |  |  |

Type of survey for compliance with Part 199 and issue of Annex VI Endorsement for domestic voyaging ships 400 GT or more (tick one)

Initial survey

Annual survey after 1 April 2023

Intermediate survey after 1 April 2023

Renewal survey after 1 April 2023

Additional survey after 1 April 2023

Type of voyages the ship will make (tick all that apply)

Domestic voyaging within NZ’s jurisdiction

Domestic voyaging beyond NZ’s jurisdiction

To undertake this survey, you must be: - a recognised surveyor with recognition category as a surveyor for MARPOL Annex VI (Part 199) for ships 400 GT or more on domestic voyages (under rule 199.680), or - a surveyor employed by a Maritime NZ Recognised Organisation delegated for Part 199 (RO). The person making (issuing) an Annex VI Endorsement is required to hold a Certificate of Surveyor Recognition issued under rule 44.22. Note: The ship must comply with Part 199 and all other relevant rules before the Certificate of Survey, New Zealand Barge Safety Certificate or Certificate of Fitness can be issued with the Annex VI Endorsement.

## Executive Summary

I am satisfied that the ship complies with all applicable Marine Protection Rules Part 199, and any applicable survey performance requirements set by the Director of Maritime New Zealand as stated in this survey report.  In respect of Part 199 requirements, the ship and its survey plan are maintained at the date of the survey.

| Signature and stamp of the surveyor |  | Date DD / MM / YYYY |
| --- | --- | --- |

Deficiencies and corrective actions

| Date | Description of deficiency and corrective action | Survey plan amended | Close out date and method |
| --- | --- | --- | --- |

The information in the table below must be recorded during the survey.

| Vessel Engine Details (complete for each engine on board) |  |
| --- | --- |
| Field | Value/Options |
| Engine type (tick option that applies) | ☐ Compression ignition ☐ Diesel-electric ☐ Electric ☐ Gas Turbine ☐ Hybrid ☐ Spark Ignition ☐ Steam ☐ Turbine |
| Year of Engine Build (YYYY) |  |
| Year of Engine Installation (YYYY) |  |
| Engine Use (tick all that apply) | ☐ Propulsion ☐ Power Generation ☐ Powering other Machinery ☐ Emergency |
| Engine Manufacturer (Name and address) |  |
| Model Number |  |
| Serial Number |  |
| Engine Approval Number (located on EIAPP) |  |
| Rated Power (kW) |  |
| Engine Speed Rating (RPM) |  |
| Number of Shafts |  |
| Fuel Type (tick all that apply) | ☐ Coal ☐ Diesel/Gas Oil ☐ Diesel/Petrol ☐ Ethanol ☐ Heavy Fuel Oil (HFO) ☐ HFO/LFO Blend ☐ Intermediate Fuel Oil (IFO) ☐ Lift Fuel Oil (LFO) ☐ Liquefied Natural Gas (LNG) ☐ Liquefied Petroleum Gas (Butane) ☐ Liquefied Petroleum Gas (Propane) ☐ Methanol ☐ Petrol |
| Sulphur level of fuel used |  |
| Required Nitrogen Oxide (NOx) Tier (I,II or III) |  |
| NOx Compliant (Yes, No or N/A) |  |
| NOx Not Applicable Reason (tick option(s) that apply) | ☐ Engine <=130kW ☐ Engine pre dates requirements ☐ Engine only used for emergencies ☐ Engine not installed |
| Proof of Compliance | ☐ EIAPP Certificate |
| Maximum Continuous Rating (MCR) |  |

| Current MARPOL details (as applicable) |  |
| --- | --- |
| Field | Value/Options |
| EEDI: Required |  |
| EEDI: Attained |  |
| EEXI: Required |  |
| EEXI: Attained |  |
| Carbon Intensity Rating (A,B,C,D or E) |  |
| Statement of Compliance (for fuel consumption reporting and Carbon Intensity)5 Issued Date (dd/mm/yyyy) |  |
| Incinerator Onboard (Yes or No) |  |
| Incinerator Compliant (Yes, No or N/A) |  |
| Date Incinerator Compliant (dd/mm/yyyy) (Date of IMO Type Approval certificate) |  |
| Ozone depleting substances (ODS) Onboard? (Yes or No) |  |
| Exhaust Gas Cleaning System (EGCS) Onboard (Yes or No) |  |

Survey Report Template - Initial survey

Annex VI Endorsement – ships 400 GT or more on domestic voyages

The survey shall ensure that the equipment, systems, fittings, arrangements and material or processes on the ship comply with the applicable requirements of Part 199.

Refer to the Surveyor Performance Requirements for Annex VI Endorsement (ships 400 GT or more on domestic voyages) for guidance when completing the survey report (www.maritimenz.govt.nz/surveyors).

| Requirements |  | Survey findings | Add photos |
| --- | --- | --- | --- |
| Documentation |  |  |  |
| 1 | Confirm that there are written procedures covering fuel change over, where applicable. Part 199 Section C4 |  |  |
| 2 | Confirm that there is for each Exhaust Gas Cleaning System (EGCS) – SOx either: - a SOx Emissions Compliance Certificate (SECC) for the EGCS - SOx, or - an Onboard Monitoring Manual (OMM) as appropriate, As well as a SOx Emissions Compliance Plan (SECP)(Regulation (Reg.) 4 of Annex VI) or approved documentation in respect of other technological means of achieving compliance. Part 199 Section C4 |  |  |
| 3 | Confirm that there is a transfer procedure, if required, for the VOC collection system. Part 199 Section C5 |  |  |
| 4 | Confirm that there are Maritime NZ approved Engine International Air Pollution Prevention (EIAPP) Certificates for each compression-ignition engine required to be certified, as described in chapter 2.1 of the NOx Technical Code. Part 199 Section C3 |  |  |
| 5 | Confirm that there is a Maritime NZ approved Technical File on board for each compression-ignition engine required to be certified. Part 199 Section C3 |  |  |
| 6 | Confirm that there is a record book or an electronic record book of engine parameters for each compression ignition engine with an EIAPP in cases where the engine parameter check method is used as a means of onboard NOx verification. (NOx Technical Code (2008), 6.2) |  |  |
| 7 | Confirm that there is an approved onboard monitoring manual for each compresssion ignition engine with an EIAPP in cases where the direct measurement and monitoring method is used as a means of onboard NOx verification (NOx Technical Code (2008), 6.4) |  |  |
| 8 | Confirm that each non-propulsion stationary/auxiliary engine required to be certified meets the relevant standard and has a manufacturer’s declaration or certificate of conformity to the applicable standard. Part 199 Section C3 |  |  |
| 9 | Confirm that each spark-ignition engine required to be certified: - meets the relevant standard and has a manufacturer’s declaration or certificate of conformity to the applicable standard, or - for engines installed before 31/12/10: - Manufacturer’s declaration or certificate of conformity to the applicable standard, or - evidence that the engine meets an appropriate standard for spark-ignition engines acceptable to the Director, or - evidence of the actual NOx emissions of the engine. Part 199 Section C3 |  |  |
| 10 | Confirm that there is an Ozone Depleting Substances record book or an electronic record book, if applicable. Part 199.365 |  |  |
| 11 | Confirm that there is a VOC Management Plan, if required. Part 199 Section C6 |  |  |
| 12 | Confirm that there is, if required, an IMO Type Approval Certificate for each incinerator on board. Part 199 Section C6 |  |  |
| 13 | Confirm that there is an instruction manual for each incinerator if required. Part 199 Section C6 |  |  |
| 14 | Confirm that there are records documenting training of the crew in operating each incinerator, if required. Part 199 section C6 |  |  |
| 15 | Confirm that there is a record of fuel changeover, where applicable, and that this record should take the form of a logbook or an electronic record book. Part 199 Section C4 |  |  |
| Nitrogen Oxide emissions from engines as from 1 January 2023 |  |  |  |
| 1 | Confirm that all compression ignition engines which are required to be certified are pre-certified in accordance with section 2.2 of the NOx Technical Code to the required Tier and installed in accordance with the approved duty cycle. Part 199 Section C3 |  |  |
| 2 | If the engine parameter check method is used: - an onboard verification survey in accordance with section 6.2 of the NOx Technical Code. Part 199 Section C3 |  |  |
| 3 | If the simplified method is used: - an onboard verification survey in accordance with section 6.3 of the NOx Technical Code. Part 199 Section C3 |  |  |
| 4 | If direct measurement and monitoring method is used (for existing ships only): - an onboard verification survey, in accordance with section 6.4 of the NOx Technical Code. Part 199 Section C3 |  |  |
| 5 | For compression-ignition engines installed on ships constructed between 1 January 1990 and 31 December 1999 with an output more than 5,000 kW and a per cylinder displacement at or above 90 litres/cylinder, check whether: - an approved method exists but is not applicable; or - an approved method is not commercially available at this survey; or - an approved method is installed and where this is the case, that there is an approved method file, and apply the verification procedures as given in the approved method file; or - the engine has been certified, confirming that it operates within the limits set forth for Tier I, Tier II or Tier III, as applicable. Part 199 Section C3 |  |  |
| 6 | For each non-propulsion stationary/auxiliary engine, and spark-ignition engine required to be certified; confirm that the engine has been installed and any maintenance carried out as per manufacturer’s manual. Part 199 Section C3 |  |  |
| 7 | NOx reducing device In cases where there are NOx reducing devices (e.g. Selective Catalytic Reduction system) on board, examine NOx reducing devices for compliance with Res.MEPC.198(62) Part 199 Section C3 |  |  |
| Ozone Depleting Substances |  |  |  |
| 1 | Confirm, if applicable, the satisfactory installation and operation of systems using ozone depleting substances. Part 199 Section C2 |  |  |
| 2 | Confirm that no installation or equipment containing ozone depleting substances has been installed after 19 May 2005, other than hydro-chlorofluorocarbons. Part 199 Section C2) |  |  |
| 3 | Confirm that no installation or equipment containing hydro-chlorofluorocarbons are fitted after 1 January 2020. Part 199 Section C2 |  |  |
| Plans and designs |  |  |  |
| 1 | Examine the arrangements for systems using ozone-depleting substances. Part 199 Section C2 |  |  |
| 2 | Examine the arrangements for NOx emission control, if applicable. Part 199 Section C3 |  |  |
| 3 | Examine the arrangements for SOx and particulate matter control, if applicable. Part 199 Section C4 |  |  |
| 4 | Examine, where applicable as an equivalence, the plans and arrangements for the exhaust gas cleaning system* or other technological methods. * Refer to the 2021 Guidelines for exhaust gas cleaning system (resolution MEPC.340(77). |  |  |
| 5 | Examine the arrangements for vapour collection systems, if applicable. Part 199 Section C5 |  |  |
| 6 | Examine the arrangements for shipboard incinerators, if applicable. Part 199 Section C6 |  |  |
| Shipboard Incinerators (installed on or after 1 January 2000) |  |  |  |
| 1 | Confirm the satisfactory installation and operation of each incinerator. Test alarms and shut-downs Check that no prohibited substances have been incinerated. Part 199 Section C6 |  |  |
| 2 | Confirm that the manufacturer's name, incinerator model number/type and capacity in heat units per hour is permanently marked on the incinerator Part 199 Section C6 |  |  |
| Sulphur Oxides and Particulate Matter |  |  |  |
| 1 | Confirm, if appropriate, that: - satisfactory arrangements are in place for using compliant fuel as required; or - satisfactory installation and operation of the fuel switching arrangements are in place when tanks are provided for different grades of fuel; or - satisfactory installation and operation of the exhaust gas cleaning system* or other technological methods are examined (MARPOL Annex VI reg.4). * Refer to the 2015 Guidelines for exhaust gas cleaning systems (resolution MEPC.259(68)) Part 199 Section C4 |  |  |
| 2 | Confirm that the In-use fuel oil sampling point(s) are fitted or designated as appropriate (For a ship constructed before 1 April 2022, In-use fuel oil sampling point shall be fitted or designated not later than the first IAPP renewal survey on or after 1 April 2023. For new builds which are constructed on or after 1 April 2022, sampling points should be designated during construction and confirmed during initial surveys.) Part 199 Section C7 |  |  |
| Volatile Organic Compounds (if applicable): |  |  |  |
| 1 | Confirm the satisfactory installation of the vapour collection piping. Part 199 Section C5 |  |  |
| 2 | Confirm the satisfactory installation and operation of the means provided to eliminate the collection of condensation in the system, such as drains in low points of the line end. Part 199 Section C5 |  |  |
| 3 | Confirm the satisfactory installation and operation of the isolation valves at the vapour manifolds. Part 199 Section C5 |  |  |
| 4 | Confirm that the ends of each line are properly identified as vapour collection lines. Part 199 Section C5 |  |  |
| 5 | Confirm that the vapour collection flanges are in accordance with the IMO guidelines and industrial standards. Part 199 Section C5 |  |  |

Initial and renewal surveys

‘Yes’ or ‘No’ to indicate whether the survey plan has been amended to reflect change in frequency of surveying the deficient item.

Close-out method refers to the method by which the surveyor will determine achievement of close-out. (eg visit or photograph)

For ships 5000 GT or more

Refer to the Guidelines for the use of electronic record books under MARPOL (resolution MEPC.312(74)).

On a ship that became a NZ ship after 1 January 2023.

Refer to the Guidelines for the use of electronic record books under MARPOL (resolution MEPC.312(74)).

Refer to the 2014 Guidelines on the approved method process (resolution MEPC.243(66)).