Periodic Survey Report Template:

Auxiliary Systems

| Survey for compliance of | Survey findings | Add photos |
| --- | --- | --- |
| Auxiliary engines |  |  |
| Auxiliary engines as approved From inspection that engines meet any specification in a ship design, construction or modification approval.) |  |  |
| Maintenance plan (From inspection of the maintenance plan for the engines) |  |  |
| Condition and maintenance (From inspection of external condition of the engines and of service, maintenance and repair records) |  |  |
| Secureness (As applicable, from inspection of the securing method and condition of attachment to main structure) |  |  |
| Operation (From test of the start-up and shut-down of the engine and of the adequacy and availability of operating instructions) |  |  |
| Monitoring and control (From inspection of the adequacy of the monitoring devices for engine oil pressure, cooling water temperature, battery charge, generator charging rate, rotational speed, and of ability to shut down engines remotely from engine space) |  |  |
| Start-up power capacity (From inspection that start-up power arrangements have sufficient capacity for multiple starts.) |  |  |
| Fuel piping (From inspection of condition and fire resistance of fuel hose) |  |  |
| Engine exhaust general (From inspection of the condition of the exhaust pipes, the noise and heat insulation, and guarding of exhaust pipes for safety of personnel.) |  |  |
| Dry exhaust discharges (From inspection of insulation limiting heat transfer into the structure, and of location of discharge clear of ventilation intakes.) |  |  |
| Wet exhaust discharges (From inspection of arrangements to prevent back-flooding and piping, and from testing any back pressure alarms.) |  |  |
| Ventilation or Heating, ventilation and air conditioning (HVAC) system |  |  |
| Operation (From test of system, including test of closing devices, fan or forced system shut down.) |  |  |
| Adequate maintenance plan (From inspection of the maintenance plan for the system) |  |  |
| Condition and maintenance (From inspection of the service, maintenance and repair records) |  |  |
| Refrigeration systems (holds and cold-stores) |  |  |
| Maintenance plan (From inspection of the maintenance plan for the refrigeration units ) |  |  |
| Condition and maintenance (From inspection of the condition of the piping, plates, coils, and insulation and of service, maintenance and repair records) |  |  |
| Operating and safety information (From inspection of, operating instructions, refrigerant data sheets, signage, and emergency procedures.) |  |  |
| Exits and ventilation (From inspection of adequacy of ventilation of and exits from the refrigerated area.) |  |  |
| Respiratory breathing apparatus (Where applicable for fishing ships, from inspection that breathing apparatus provided meets rule requirements.) |  |  |
| Alarms (From inspection that cold-store has required warning light, external alarm, internal means of sounding the alarm, door location and release mechanism; and from testing the alarm) |  |  |
| Compressed Air Systems |  |  |
| Maintenance plan (From inspection of the maintenance plan for the air compressors and related equipment.) |  |  |
| Condition and maintenance (From inspection of the condition of the compressors, pipework and receivers and of the service, maintenance and repair records) |  |  |
| Operation (From running the compressor and checking for extensive air leakage) |  |  |
| Compressed air security (From inspecting the fuel/power supply arrangements for independence and capacity) |  |  |
| Condensate drainage (From inspection that receiver has valve for drainage of accumulated condensate) |  |  |
| Air piping (From inspection of piping for damage, constrictions and leakage) |  |  |
| Air quality (From inspection that air intake location is suitable for quality of compressed air required) |  |  |
| Pressure vessel certification (From inspection of pressure vessel certification documentation.) |  |  |