Periodic Survey Report Template

Fluid transfer systems

Covering bilge, ballast, fuel, lube oil, potable water and salt water systems

| Survey for compliance of: | Survey findings | Add photos |
| --- | --- | --- |
| Bilge systems |  |  |
| Location of bilge suctions (From inspection that bilge piping is arranged so as to meet drainage requirements when ship is heeled.) |  |  |
| Piping and hoses (From inspection of pipes and hoses for their condition and protection from damage, and from sighting of evidence of their capability to work effectively under the required suction pressure.) |  |  |
| Back-flooding prevention (From inspection that the bilge system is arranged to prevent back-flooding, including located of on-return devices.) |  |  |
| Watertight integrity of compartments (From inspection that bilge system is arranged to prevent water passing from a flooded watertight compartment into other compartments.) |  |  |
| Bilge distribution manifold (Where applicable, from inspection that vessel’s bilge distribution manifold is accessible and has non-return valves in the bilge distribution box.) |  |  |
| Bilge sounding arrangements (Where applicable, from inspection and testing of sounding arrangements, including closing cock in propulsion machinery space.) |  |  |
| Strainers, strum boxes, mud boxes (From inspection of bilge suction strainers, strum boxes and mudboxes for corrosion, damage and strainer hole size.) |  |  |
| Collision bulkhead drainage (Where applicable, from inspection of the condition, location and accessibility of collision bulkhead drainage valve.) |  |  |
| Float switches protection (From inspection that float switches on pump and/or alarm are protected from jamming caused by bilge debris.) |  |  |
| Visual bilge pump alarm (From inspection that submersible bilge pumps are linked to a visual alarm at the steering position to indicate when pump is running.) |  |  |
| Audible bilge level alarm (From inspection that float is linked to an audible alarm located near the steering position, and from testing that it is operable at all times when there is any person on board.) |  |  |
| Alternative arrangements (Where applicable, from inspection that any alternative arrangements for small compartments have appropriate buoyancy media properly secured in place; and easily removable for inspection of the void space.) |  |  |
| Bilge Pumps |  |  |
| Number, type and capacity (From inspection of the number, type and total capacity of the bilge pumps relative to applicable rule requirements, and from reviewing evidence of delivery/discharge performance.) |  |  |
| Manual (hand operated) bilge pumps (From inspection of the condition and location of any manual bilge pumps and from testing their operation.) |  |  |
| Electric bilge pumps (From inspection of the condition and location bilge pumps, including submersible pumps, and from testing their operation.) |  |  |
| Installation and accessibility (From inspection of the secureness of the pumps’ installation and of their accessible for inspection, removal and maintenance.) |  |  |
| Power sources (From inspection that the bilge pumps have the required number of power sources with required power capacity.) |  |  |
| Portable pumps (From inspection of the suitability, condition and storage location or any portable bilge pumps and from testing their operation.) |  |  |
| Pumps condition and maintenance (From inspection of the condition of the pumps and motors and from review of their service and maintenance records.) |  |  |
| Fuel filling, storage and transfer systems |  |  |
| Fuel filling facilities (From inspection that the outside engine spaces and signage meet requirements.) |  |  |
| Fuel transfer system (From inspection the at the pump type, filters, emergency shut off facilities and signage meet requirements.) |  |  |
| Fuel pumps and motors (From inspection that the pump motors have the required methods of remote emergency stopping controls.) |  |  |
| Non-portable fuel tanks (As applicable for the flashpoint of the fuel concerned, from inspection of the suitability of the tanks, including their venting, openings, baffles, contents measurement and support structures.) |  |  |
| Portable fuel tanks (Where applicable, from inspection of the condition and secureness of the tanks and of any related fuel lines.) |  |  |
| Testing and maintenance (From inspection of the pressure testing and general maintenance records of the tanks, and the filling and transfer systems.) |  |  |
| Fresh or potable water |  |  |
| Capacity and labelling (From inspection that the vessel has the required capacity of tanks for potable water, labelled accordingly.) |  |  |
| Segregation (From inspection that the potable water tanks are adequately segregated from on board tanks containing liquids.) |  |  |
| Cleaning access and drainage (From inspection that the tanks include adequate internal access for inspection, cleaning and complete drainage.) |  |  |
| Pumping systems, pipes and filters (From inspection that all potable water pumps, pipes and filers are in clean and working condition.) |  |  |
| Service and maintenance (From inspection of the servicing, cleaning, and maintenance records of the systems.) |  |  |
| Ballast water system |  |  |
| Pumping system integrity (From inspection of the ballast pumping arrangement, in terms of protection of the vessel’s watertightness.) |  |  |
| System condition (From inspection of the condition of the ballast pumps, piping and valves.) |  |  |
| Ballast tank suitability (From inspection of the suitability of each tank’s means of filling, venting, sounding and emptying.)” |  |  |
| Instructions and signage (From inspection of the adequacy of the operating instructions and signage for the valves and overall system.) |  |  |
| Service and maintenance (From inspection of the servicing and maintenance records of the systems) |  |  |
| Sea water system |  |  |
| Pumping system integrity (From inspection of the ballast pumping arrangement, in terms of protection of the vessel’s watertightness.) |  |  |
| System condition (From inspection of the condition of the pumps, piping and valves.) |  |  |
| Service and maintenance (From inspection of the servicing and maintenance records of the systems.) |  |  |
| Lubricating oil systems |  |  |
| Lube oil systems installation (From inspection that the systems’ pumps, piping and fittings are securely installed and present minimal danger to persons on board the vessel.) |  |  |
| System condition (From inspection of the condition of the lube oil pumps, piping, valves and filters, including inspection for any leakage.) |  |  |
| Storage tanks (From inspection of the condition of the storage tanks, including shut-off valves and signage.) |  |  |
| Purifiers (From inspection of the external condition of the oil purifiers.) |  |  |
| Servicing and maintenance (From inspection of the service and maintenance records of the lube oil system including the purifiers.) |  |  |