Periodic Survey Report Template:

Hull decks and superstructure – FRP ships

| Survey for compliance of: | Survey findings | Add photos |
| --- | --- | --- |
| Hull External |  |  |
| Structural integrity – distortion (From viewing hull profile and inspection of causes of any visible distortion or deformation) |  |  |
| Structural integrity – laminate (From inspection of laminate for blisters, voids, cracks, delamination and any other form structural weaknesses) |  |  |
| Laminate protection (From inspection of the antifouling and laminate coating protection systems and sacrificial anodes) |  |  |
| Platforms integrity (From inspection of beltings, anchor platforms and boarding platforms for signs of damage and moisture ingress) |  |  |
| Fittings and laminate integrity (From inspection of all fittings and surrounding hull areas for damage, corrosion, fastenings failure and laminate degradation.) |  |  |
| Outboard motor mountings (Where applicable, from inspection of mounting and surrounding structures for damage, cracking, movement or distortion and secureness.) |  |  |
| External ballast (Where applicable, from inspection for secureness on the hull and for signs of deterioration on the ballast or fastenings.) |  |  |
| Portlights integrity (From inspection of secureness and sealing of the portlights, their materials and the fastenings.) |  |  |
| Inlets and discharges watertightness (From inspection of all inlets and discharges, including related valves and seacocks, for watertightness and weathertightness.) |  |  |
| Submerged fittings secureness (When surveying the hull out of water, from inspection of the condition and secureness of any sea chests and of fittings related to the propulsion and steering systems.) |  |  |
| Hull Internal |  |  |
| Structural integrity – general (From inspection of laminate for blisters, cracks, delamination, moisture ingress and any other form of structural weaknesses) |  |  |
| Structural integrity – voids (From inspection of lockers, storage places and other voids for cracks, leakage and delamination.) |  |  |
| Structural integrity – internal bulkheads (From inspection of internal bulkheads and partitions for damage, cracks, shatter marks, compression damage , and deterioration of secondary bonding of internal bulkheads, girders, floors and frames) |  |  |
| Bilge compartment integrity (From inspection of internal bilge compartments for damage, cracks, and oil-water indications, and for blocked limber holes.) |  |  |
| Internal mountings for external fittings (From inspection of mountings, backing blocks and fastenings related to external fittings, for corrosion and leakage.) |  |  |
| Fastenings and through-bolts (From test of plate bolts, keel bolts and other through-bolts fastenings for soundness, including measurement for damage or wastage as applicable.) |  |  |
| Internal door closure mechanisms (From inspection and testing of hinge and securing mechanisms of internal access doors.) |  |  |
| Deck hatch effectiveness (From inspection of internal area around deck hatches for cracks and leakage, and for the condition and operation of securing dogs, gaskets and hatch drains.) |  |  |
| Engine and gearbox mountings (For jet and other inboard units, from inspection of mountings and surrounding areas for damage, cracks, excessive movement and wear.) |  |  |
| Intake strainer damage (For jet powered boats, from inspection of intake strainer, mounting flange and surrounding hull for impact damage, cracks and water ingress.) |  |  |
| Drive mountings integrity (For stern drive and surface drive ships, from inspection of the drive mountings supporting structures and joins and fastening bolts for corrosion, cracking and water ingress.) |  |  |
| Stern tubes integrity (Where applicable, from inspection of stern tubes, shaft log and rubber tube mountings and surrounding structures for corrosion and leakage.) |  |  |
| Portlights and deadlights effectiveness (From inspection of the hull portlights for secureness, for operation of portlight and deadlight closures and dogs, and for overall weather- or watertight effectiveness.) |  |  |
| Posts, bollards and masts bonding (Where applicable, from inspection of where posts, towing bollards or masts are bonded to strength members, for secureness, loose fittings and damage.) |  |  |
| Decks and superstructure |  |  |
| Structural integrity – distortion (From inspection for distortion or deformation of cabin and superstructure profile.) |  |  |
| Structural integrity – laminate (From inspection of mouldings, high load areas of decks and cabin structure for damage, delamination or cracking.) |  |  |
| Weathertightness – windows (From inspection of windows, skylights and screens, including any shutters, deadlights and stormcovers, for secureness and weathertightness.) |  |  |
| Water- and weathertightness - doors (From inspection of all doors, door seals surrounding structures, sills, hinges, latches and securing dogs for damage and watertightness or weathertightness, as applicable.) |  |  |
| Water- and weathertightness – hatches (From inspection and testing of deck hatch covers, coamings, gaskets, hinges, latches and securing mechanisms for watertightness or weathertightness, as applicable.) |  |  |
| Water- and weathertightness –pipes (From inspection of height and coamings of all air pipes, ducts and ventilators and of the weathertightness of their closures and fastenings) |  |  |
| Manholes (From inspection of adequacy of manholes openings, and of weathertightness of covers and fastening mechanism.) |  |  |
| Machinery space openings (From inspection of the condition of the casings, and of compliance of sill heights.) |  |  |
| Integrity at mast attachments (Where applicable, from inspection of attachment of any mast foot on the deck for damage in the surrounding deck structure, and for weathertightness at entry of mast through the deck.) |  |  |
| Integrity – high load areas of deck (From inspection of deck winch mounting structures for movement, compression damage and wear and tear on hull laminate in impact zone.) |  |  |
| Deck surface safety (From inspection of the surface of the deck and any cockpit sole for slip resistance.) |  |  |
| Deck and superstructure ladders safety (From inspection of the deck and superstructure ladders, for the safety of their tread and their fastenings to the ship structure) |  |  |
| Watertightness – fitting holes (From inspection for any unused unfilled unused holes and loose fastenings that will allow water egress into buoyancy compartments). |  |  |
| Water freeing arrangements (From inspection of all water freeing arrangements, including cockpit drains, duckbill drains and chain lockers, and of the operation and effects of covers, grids and flaps on water freeing.) |  |  |
| Stability |  |  |
| Stability of vessel considered (From verification of any major modification, changes to structure or equipment (eg new, replaced or relocated) which might affect stability) |  |  |
| Stability book (if applicable) (From inspection, the stability book is approved and reflects current loading pattern, configuration, etc of the boat.) |  |  |
| Guardrails and bulwarks |  |  |
| Guardrail secureness (From inspection of condition of guardrails and handrails and from test of the secureness of the stanchions, and their mountings and fastenings.) |  |  |
| Guard wires (From inspection of the condition of any wires, and where flexible wires are used as lifelines from inspection of the condition and tightness of siblings.) |  |  |
| Heights and spacing (From inspection of height of guard rails and bulwarks and spacing between guard rails and/or courses) |  |  |
| Bulwark condition and security (From inspection of condition of bulwarks for damage or other sources of insecurity) |  |  |
| Toerail condition (Where applicable, from inspection of the condition of the toerail for damage and secureness) |  |  |
| Safe navigation (From inspection that sighting for safe navigation of the ship is not impeded by the guardrails or bulwarks) |  |  |
| For pontoon, collar and RIB vessels only |  |  |
| Integrity of rigid tubes (Where applicable, from inspection of the welding, soundness and watertight integrity of the tubes/compartments) |  |  |
| Integrity of inflatable tubes (Where applicable, from inspection of the condition of the tube compartments for soundness and no leakage) |  |  |
| Tube inflation and pressure control (Where applicable, from inspection of condition of non-return and pressure relief valves and means of deflation) |  |  |
| Protection of tubes (Where applicable, from inspection of the condition and effectiveness of rubbing strips and covers) |  |  |
| Bonding of hull and tubes (Where applicable, from inspection of the secureness of the bonding at the bow and sides.) |  |  |
| Patches for fittings (From inspection of the existence and effectiveness of patches provided for attachment of fittings) |  |  |
| Secureness of fittings (From inspection of the secureness of attachment of fittings, including windscreen and towing eyes.) |  |  |