Periodic Survey Report Template:

Hull decks and superstructure – Wooden ships

| Survey for compliance of: | Survey findings | Add photos |
| --- | --- | --- |
| Hull External |  |  |
| Structural integrity – distortion (From viewing hull profile and inspection of causes of any visible distortion or deformation) |  |  |
| Structural integrity – hull material (From inspection and suitable test for damage, defects, cracks, marine borer, decay, ineffective caulking and any other form of weakness or lack of watertightness.) |  |  |
| Full-hull skin integrity (Where applicable, eg where FRP skin is over un-caulked planks, from inspection of the bonding and of skin watertightness) |  |  |
| Protective sheathing (Where applicable, eg where used over hull areas prone to wear or damage, from inspection for damage, loose fastenings and marine borer attack that make sheathing ineffective.) |  |  |
| Protection systems (From inspection of the antifouling and metal protection coating systems, including sacrificial anodes.) |  |  |
| Secureness of hull fastenings (From suitable tests for tightness and wastage, of a sample of the hull fastenings.) |  |  |
| Keel integrity (From inspection of keel, including any sacrificial keel and shoes, for cracks, marine borer and hogging or sagging.) |  |  |
| Sheer planks integrity (From inspection of sheer planks for decay under beltings)' |  |  |
| External ballast (Where applicable, from inspection for secureness on the hull and for signs of corrosion or other deterioration on the ballast, fastenings and as required, the hull under the ballast.) |  |  |
| Stem and deadwoods integrity (From inspection for damage, cracks, marine borer and decay, and that the interfaces between timber and planking are secure and watertight.) |  |  |
| Platforms integrity (From inspection of beltings, anchor platforms and boarding platforms and fastenings for signs of damage and cracking) |  |  |
| Fittings and hull integrity (From inspection of all fittings and surrounding hull areas for damage, corrosion, fastenings failure and decay.) |  |  |
| Outboard motor mountings (Where applicable, from inspection of mounting and surrounding structures for damage, corrosion, movement or distortion, and secureness.) |  |  |
| Portlights integrity (From inspection of secureness and sealing of the portlights, their materials and the fastenings.) |  |  |
| Inlets and discharges watertightness (From inspection of all inlets and discharges, including related valves and seacocks, for watertightness and weathertightness.) |  |  |
| Submerged fittings secureness (When surveying the hull out of water, from inspection of the condition and secureness of any sea chests and of fittings related to the propulsion and steering systems.) |  |  |
| Hull Internal |  |  |
| Structural integrity – general (From inspection of hull surfaces for damage, cracks, decay, water ingress.) |  |  |
| Structural integrity – stem, apron, breasthooks, knees and bollards (From inspection for cracks, marine borer, decay, electrical or galvanic action and other deterioration, and from sample testing fastenings for wasting or necking.) |  |  |
| Structural integrity – gunwale, stringers, beamshelf, deckbeams, frames and ribs (From inspection for cracks, marine borer, decay, electrical or galvanic action and other deterioration.) |  |  |
| Structural integrity – voids (From inspection of lockers, storage places and other voids for cracks, decay and leakage.) |  |  |
| Structural integrity – internal bulkheads (From inspection of internal bulkheads, stiffeners and partitions for damage, for cracks, marine borer, decay, electrical or galvanic action and other deterioration.) |  |  |
| Bilge compartment integrity (From inspection of internal bilge compartments for damage, cracks, and oil-water indications, and for blocked limber holes.) |  |  |
| Internal mountings for external fittings (From inspection of mountings, backing blocks and fastenings related to external fittings, for cracks, decay, corrosion and leakage.) |  |  |
| Fastenings and through-bolts (From test of plate bolts, keel bolts and other through-bolts fastenings for soundness, including measurement for damage or wastage as applicable.) |  |  |
| Internal door closure mechanisms (From inspection and testing of hinge and securing mechanisms of internal access doors.) |  |  |
| Deck hatch effectiveness (From inspection of internal area around deck hatches for cracks and leakage, and of the condition and operation of securing dogs, gaskets and hatch drains.) |  |  |
| Engine and gearbox mountings (For jet and other inboard units, from inspection of mountings and surrounding areas for damage, cracks, excessive movement and wear.) |  |  |
| Intake strainer damage (For jet powered boats, from inspection of intake strainer, mounting flange and surrounding hull for impact damage, cracks and water ingress.) |  |  |
| Drive mountings integrity (For stern drive and surface drive ships, from inspection of the drive mountings supporting structures and joins and fastening bolts for corrosion, cracking and water ingress.) |  |  |
| Stern tubes integrity (Where applicable, from inspection of stern tubes, shaft log and rubber tube mountings and surrounding structures for corrosion and leakage.) |  |  |
| Portlights and deadlights effectiveness (From inspection of the hull portlights for secureness, for operation of portlight and deadlight closures and dogs, and for overall weather- or watertight effectiveness.) |  |  |
| Posts, bollards and masts bonding (Where applicable, from inspection of where posts, towing bollards or masts are bonded to strength members, for secureness, loose fittings and damage.) |  |  |
| Decks and superstructure |  |  |
| Structural integrity – distortion (From inspection for distortion or deformation of cabin and superstructure profile.) |  |  |
| Structural integrity – deck material (From inspection for deficiencies, damage, decay, cracks, and unsound caulking and fastenings. that affect deck integrity or allow water egress to the hull interior.) |  |  |
| Integrity – high load areas of deck (From inspection of high load areas, including mountings of anchor or deck winches, gantrys, outboard well, and cockpit, for damage, decay or cracking.) |  |  |
| Weathertightness – windows (From inspection of windows, skylights and screens, including any shutters, deadlights and stormcovers, for secureness and weathertightness.) |  |  |
| Water- and weathertightness - doors (From inspection of all doors, door seals surrounding structures, sills, hinges, latches and securing dogs for damage and watertightness or weathertightness, as applicable.) |  |  |
| Water- and weathertightness – hatches (From inspection and testing of deck hatch covers, coamings, gaskets, hinges, latches and securing mechanisms for watertightness or weathertightness, as applicable.) |  |  |
| Water- and weathertightness –pipes (From inspection of height and coamings of all air pipes, ducts and ventilators and of the weathertightness of their closures and fastenings) |  |  |
| Manholes (From inspection of adequacy of manholes openings, and of weathertightness of covers and fastening mechanism.) |  |  |
| Machinery space openings (From inspection of the condition of the casings, and of compliance of sill heights.) |  |  |
| Effects from mast attachments (Where applicable, from inspection of attachment of any mast foot on the deck for damage in the surrounding deck structure, and for weathertightness at entry of mast through the deck.) |  |  |
| Deck surface safety (From inspection of the surface of the deck and any cockpit sole for slip resistance.) |  |  |
| Deck and superstructure ladders safety (From inspection of the deck and superstructure ladders, for the safety of their tread and their fastenings to the ship structure) |  |  |
| Water freeing arrangements (From inspection of all water freeing arrangements, including cockpit drains, duckbill drains and chain lockers, and of the operation and effects of covers, grids and flaps on water freeing.) |  |  |
| Stability |  |  |
| Stability of vessel considered (From verification of any major modification, changes to structure or equipment (eg new, replaced or relocated) which might affect stability) |  |  |
| Stability book (if applicable) (From inspection, the stability book is approved and reflects current loading pattern, configuration, etc of the boat.) |  |  |
| Guardrails and bulwarks |  |  |
| Guardrail secureness (From inspection of condition of guardrails and handrails and from test of the secureness of the stanchions, and their mountings and fastenings.) |  |  |
| Guard wires (From inspection of the condition of any wires, and where flexible wires are used as lifelines from inspection of the condition and tightness of siblings.) |  |  |
| Heights and spacing (From inspection of height of guard rails and bulwarks and spacing between guard rails and/or courses) |  |  |
| Bulwark condition and security (From inspection of condition of bulwarks for damage or other sources of insecurity) |  |  |
| Toerail condition (Where applicable, from inspection of the condition of the toerail for damage and secureness) |  |  |
| Safe navigation (From inspection that sighting for safe navigation of the ship is not impeded by the guardrails or bulwarks) |  |  |