Periodic Survey Report Template

Propulsion and Steering systems

| Survey for compliance of | Survey findings | Add photos |
| --- | --- | --- |
| Propulsion. Inboard, including stern drive and waterjet engines |  |  |
| Engines as approved (From inspection that engines installed are as per design, construction or modification approval.) |  |  |
| Engine maintenance plan (From inspection of the maintenance plan for the engine) |  |  |
| Engine condition and maintenance (From inspection of the external condition of the engine and of the maintenance and servicing records.) |  |  |
| Secureness (From inspection of the securing method and condition of mounts and attachment to the main structure.) |  |  |
| Starting arrangements (From inspection that the starting arrangements have sufficient capacity for multiple startings.) |  |  |
| Emergency starting arrangements (From inspection that the emergency starting arrangements have adequate independent capacity.) |  |  |
| Engine start and stop control (From inspection that the engine can be started and shut down externally of the engine space and without opening the engine box.) |  |  |
| Operational (From test of the start-up and shut-down of the engine) |  |  |
| Operating instructions (From inspection of adequacy and availability of operating instructions, including for remote and automatic shutdown.) |  |  |
| Engine and gearbox monitoring (From inspection of the adequacy of monitoring devices for engine and gearbox oil pressures, cooling water temperature, rotational speed, and generator charge rate.) |  |  |
| Engine cooling (From inspection of the condition of the engine cooling system.) |  |  |
| Alarms (From inspection of the adequacy of engine and gearbox audible and visual alarms.) |  |  |
| Engine boxes fire resistance (From inspection of the fire resistance of the box around the engine) |  |  |
| Ventilation interlock (From testing that interlocks of the pre-starting ventilation and the exhaust blowers with the engine ignition switch.) |  |  |
| Fuel system (From inspection that the fuel line piping and flexible hoses are in sound condition and meet type requirements.) |  |  |
| Engine exhaust system |  |  |
| System general (From inspection of the condition of the exhaust pipes, the noise and heat insulation, and guarding of exhaust pipes for safety of personnel.) |  |  |
| Dry exhaust discharge (From inspection of the location of the discharge relative to air intakes, and of condition of insulation limiting transfer of heat into the vessel structure.) |  |  |
| Wet exhaust discharge (From inspection of arrangements to prevent back-flooding, including testing any alarms if fitted.) |  |  |
| Shut-off and non-return valves (From inspection of the condition, accessibility and signage for exhaust system valves.) |  |  |
| Propulsion. Outboard engines |  |  |
| Engines as approved (From inspection that engines attached are as per design, construction or modification approval.) |  |  |
| Engine maintenance plan (From inspection of the maintenance plan for the engine.) |  |  |
| Engine condition and maintenance (From inspection of the external condition of the engine and of the maintenance and servicing records.) |  |  |
| Secureness (From inspection of the securing method and condition of attachment to main structure.) |  |  |
| Propulsion. Shaft drive, including azimuthing propellers, and other systems |  |  |
| Shaft condition (From inspection critical areas of the shaft for wear and corrosion, including bearings, shaft sleeves, tapers, couplings and retaining nuts, and from measurement of shaft diameter when applicable.) |  |  |
| Shaft support (From inspection of the shaft support brackets, shaft overhang, stern bearings, stern tubes and seals.) |  |  |
| Propeller or nozzles (From inspection of the condition of the propeller or nozzles, as applicable.) |  |  |
| Drive system maintenance plan (From inspection of the maintenance plan for the drive system.) |  |  |
| System maintenance record (From inspection of the maintenance and servicing records of the drive system.) |  |  |
| Propulsion. Sailing systems |  |  |
| Secureness – keel stepped (From inspection of partners or mast chocks, the boot around mast gate, supporting beam fastenings and base, wiring connections and mast drain) |  |  |
| Secureness – deck stepped (From inspection of the structure around mast tabernacle, supporting beam fastenings and base, wiring connections and mast drain.) |  |  |
| Masts, spars and booms – steel or aluminum (From inspection for thin or corroded areas, diminution, pitting and oxidation, wear and cracking on fastenings and fittings.) |  |  |
| Masts, spars and booms – FRP (From inspection for thin or corroded areas, delamination, blisters, pitting and oxidation on fastenings and fittings.) |  |  |
| Masts, spars and booms – Wood (From inspection for rot, fungal, coatings, for pitting and oxidation on fastenings and fittings.) |  |  |
| Spreaders (From inspection for corrosion or rot, and of the spreader bases or sockets for mast compression.). |  |  |
| Masthead (From inspection for cracks in welds, and for corrosion and wear of tangs, fittings, pins and wiring , and for elongation or distortion) |  |  |
| Standing and running rigging (From inspection of rigging, shrouds and stays, for broken wire strands, bending on swage barrels, an cracking of rod heads.) |  |  |
| Chain plates and other securing devices (From inspection of condition of chain plates, turnbuckles and fastenings, for pin holes, damage and wear.) |  |  |
| Sails condition (From inspection inspect for damage from UV light, chafing manhandling and flogging, and from testing by hoisting and sheeting.) |  |  |
| Sails sufficiency (From inspection that vessel has sufficient sails to meet rule requirements.) |  |  |
| Halyards, sheets and cleats (From inspection of the condition of these and other methods of cleating or tying off.) |  |  |
| Working aloft and deck safety (Where access to the rig is an operational necessity, from inspection of safety of the arrangements for persons to work aloft and out on any bowsprit.) |  |  |
| Riggers report (As applicable, from inspection that the vessel has a full riggers report that is not less than five years old.) |  |  |
| Steering systems |  |  |
| Rudder (From inspection of the rudder and fastenings for corrosion, defects, cracking and diminution.) |  |  |
| Rudder stock (From inspection, and measurement as applicable, of the rudder stock including key ways, keys, couplings and bolts. for corrosion, defects, cracking and diminution.) |  |  |
| Rudder tube (From inspection of the rudder tube or trunking for corrosion, defects, cracking and diminution.) |  |  |
| Gland packing or seal (From inspection of the gland packing for corrosion or defects.) |  |  |
| Pintle (From inspection, and measurement as applicable, of the pintle and pintle bearings and bushes for wear and defects.) |  |  |
| Tiller arm or quadrant (From inspection of the condition of the tiller arm or quadrant, and the secureness of fixing to the rudderstock.) |  |  |
| Rudder movement and stops (From inspection that rudder has required port and starboard movement, and that the rudder stops are effective.) |  |  |
| Helm and rudder turn (From inspection that the rudder and helm turn in the same direction and that any rudder position indicator fitted works satisfactory.) |  |  |
| Rudder support (From inspection, and measurement where applicable, of the rudder support for wear at critical areas.) |  |  |
| Spade rudder carrier (From inspection, and measurement where applicable, of the spade rudder carrier and upper bearing for wear at critical areas.) |  |  |
| Upper rudder stock bearings (From inspection, and measurement where applicable, of the upper rudder stock bearings for wear at critical areas.) |  |  |
| Rods, cables, chains and hydraulics (From inspection of the condition of the mechanical and hydraulic systems components transmitting steering from the helm to the rudder.) |  |  |
| Steering nozzles (From inspection of welds, plating, webs, fins, nozzle stock and heel pintle for defects, cracking or corrosion.) |  |  |
| Secondary means of steering (Where applicable, from inspection and testing of the working condition of the secondary means of steering.) |  |  |
| Steering system maintenance (From inspection of the external condition of the engine and of the maintenance and servicing records.) |  |  |
| Operation at-sea – combined engine and drive systems |  |  |
| Engine and gearbox secureness (From sea trial test of the engine, and gearbox where applicable, for secureness and excessive movement on mountings, when powered ahead and astern.) |  |  |
| Steering (From sea trial of the effectiveness of the steering when powered ahead and astern.) |  |  |